Pre-Thesis

In fulfillment of the requirements for the PhD candidacy examination

Optimal deployment of static sensors for industrial leak detection using a Multiobjective Evolutionary Algorithm (MOEA)

בעברית:

Submitted by:

Idit Belachsen

Faculty of Civil and Environmental Engineering Technion-Israel Institute of Technology

31.07.2019

Advisors: Associate Professor Barak Fishbain, Associate Professor Shai Kendler

Abstract

TBD.

Introduction

Degraded air quality is a consequence of heightened emissions from a wide range of pollution sources, both anthropogenic, such as transportation or industry and natural, such as dust plumes, wildfires or vegetation. Dense urban centers and industrialized regions serve as “hotspots” in a continuum of concentrations, dictated by the rate by which all emitted substances undergo transport, diffusion, chemical transformation and deposition to the ground (Atmospheric chemistry and physics). Fossil fuel combustion processes that govern todays’ industrial and transportation activities are major emitters of gaseous pollutants to the troposphere, mainly sulfur dioxide (SO2), nitrogen oxides (NOx, nitrogen monoxide (NO) and dioxide (NO2)), carbon monoxide (CO) and dioxide (CO2), volatile organic compounds (VOCs) and others. Particulate pollutants, known as particulate matter (PM), are emitted as well in combustion processes and can be divided to primary particles (such as black carbon (BC)) and secondary particles (like sulfates and nitrates) which are formed in the atmosphere by oxidation of primary gaseous pollutants. (coarse/fine/ultrafine particles?) Another secondary pollutant is the ozone (O3). Like other secondary pollutants, it is produced naturally in the troposphere by photochemical oxidation of primary pollutants by the hydroxyl radical (OH). Many other anthropogenic pollutants are emitted from various industrial processes, among them are X that is emitted from Y, X1 from Y1.

The main incentive to reduce and control emission rates from anthropogenic sources is of course insuring (assuring?) population health. World health organization (WHO) estimates that 4.2 million premature deaths globally are linked to ambient air pollution, mainly from heart disease, stroke, chronic obstructive pulmonary disease, lung cancer, and acute respiratory infections in children. Pollutants with the strongest evidence for public health concern include particulate matter (PM), ozone (O3), nitrogen dioxide (NO2) and sulfur dioxide (SO2) (WHO,2019).

Epidemiologic studies (others as well?) try to evaluate (assess?) past and present population exposure to air pollution and correlate the level of exposure to observed health effects in the population. The major challenge in this type of work is in producing accurate pollution concentration maps of high spatial and temporal resolution that can enable finding such correlations at a personal level (e.g. estimating concentrations in the exact place of residence of a subject). These studies rely on data of ambient pollution concentrations usually obtained by two methods; i) routine measurements reported by standard air quality monitoring (AQM) stations, ii) short-term measurement campaigns which usually utilize large number of sensors. Evaluations using the first method are considered very accurate, since AQM stations are equipped with expensive, pollutant-designated measuring devices, that are maintained and calibrated on a regular basis by regulatory authorities. However, these tend to suffer from a few apparent flaws; i) their span is usually sparse, and cannot represent well the spatial and temporal variability of a typical pollutant (https://www.ncbi.nlm.nih.gov/pmc/articles/PMC2233947/), ii) samples of air are taken a few meters above street level and hence cannot represent well the extent of exposure of a passerby, and iii) they are costly to maintain. Short-term measurement campaigns on the other hand can provide a higher spatial resolution of the desired region of research, but for a limited time span only, that is rarely adequate for a comprehensive epidemiological study.

WDESN… + CITIZEN SCIENCE

In the past couple of years, citizen science projects had become more common, and large amounts of air quality data are being collected today by individuals, usually using low-cost sensors of various types. Using such data however requires an exhaustive preprocessing work as reliability of some of the measurements might be questionable.

Spatial interpolation methods (Kriging, inverse distance weighting (IDW), spline) may help overcome sparse representation of concentration and can be used to produce pollution dense maps of certain locations and times. Land use regression (LUR) models can link environmental variables (e.g., road type, traffic volume, topography, land cover) with measurements in monitoring locations and then use these variables as model predictors at unmonitored locations, and possibly as predictors in time as well (?). Nevertheless, atmospheric transport and dispersion (ATD) models are the only models that can quantify the deterministic relationships between sources’ emissions and concentrations in space. ATD models can forecast the spread of the pollutants, when available results from monitoring data are used for calibration and evaluation of model performance. Population exposure estimation is one possible application of ATD models. Oftentimes, these models serve as a useful tool for regulatory authorities to assess baseline ambient concentrations, analyze the relative importance of various emission sources, or test emission reduction strategies (Stein, Ariel F).

For an accurate forecast, several variables are needed as input to ATD models, including, among others: meteorological data, the strength of the release and its location. While meteorological data of reasonable resolution (of a few square kilometers and 15-30 minutes) are usually available from local weather stations or weather models, the strength, time or even location of the release of the pollutant are frequently unknown. As a consequence, sensor measurements are often being used to determine source parameters in an inverse modelling problem of source-term estimation (STE) (review paper). In these methods, source parameters (sent as input to an ATD model) are modified until the difference between the calculated and observed measurements is minimal. A certain optimization technique is usually applied in order to search the solution space and reach an accurate evaluation. Once the source term is calculated, it is used as input to an ATD model that can generate high resolution concentration maps.

As mentioned above, the momentary rate in which pollution is emitted from most sources is usually unknown. In Israel for example, an inventory of pollutants released to the environment by all industrial businesses (production plants?) is produced by the ministry of environmental protection on a yearly resolution, and is based mainly on the plant’s statement. Some plants possess monitoring equipment that enables high resolution evaluation of pollutant emission rates, but the majority (?) rely on occasional samplings which only enable the calculation of a ratio between the emission rate of the pollutant and the production rate during the time of the sampling. Most plants are given a certain emission permit that refer to a certain average emission rate. Certain standard deviations are allowed, with a unique value for each pollutant. Enforcement of these permits relies on the ministry’s inspection.

The motivation behind estimating source parameters in an adequate temporal resolution is now clear. Regulatory authorities may wish to enforce plants that temporarily exceed their emission permits, reveal leaks or new unknown sources. Epidemiological studies may wish to account for high temporal variability of pollution concentrations since health impacts might also be triggered by short-term exceedances of pollution concentrations as well as long term exposure. Such exceedances might be missed in studies that focus on long-term continuous exposure to average daily, monthly or yearly pollution levels.

Nevertheless, estimating source parameters is not an easy task. As the number of sources increases, the complexity of the problem increases and a rigorous deployment of a network of sensors is needed to provide the right representation of the contamination levels in the field. Measurements achieved by this network…. Many approaches exist to what is the best way to span such a network, considering the limitations and constrains (e.g. cost, location, human resources available) that characterize the region. These will be reviewed thoroughly in section X.

The following proposed work offers a modelling tool that can serve stakeholders when either planning the positioning of a stationary network of sensors or planning a span of sensors during a routine sampling task. It is based on meteorological data (wind speed and direction and atmospheric stability) and on a quantitative measure of the complexity of the given set, making it possible to evaluate how hard it may be to separate pollution plumes that overlap. Optimization….

Research objectives

TBD.

Research contribution

TBD.

Literature review

Sensors placement, WDESN, optimization methods, dispersion model types used?, evolutionary algorithms

Methods and Research plan

- The Gaussian plume model

Atmospheric transport and dispersion (ATD) modeling refers to the mathematical description of pollutant transport in the atmosphere. The term dispersion is comprised of diffusion (due to turbulent eddy motion) and advection (due to wind) that occurs within the air near the Earth’s surface [1]. Several types of fundamental dispersion models exist: box models, Gaussian plume models, Lagrangian models, Eulerian dispersion models and Dense gas models as well as extensions and combinations of all of the above [2].

The Gaussian plume model is one of the simplest and widely used models that offers an analytical solution to the advection-diffusion equation for idealized circumstances, corresponding to a continuous point source that emits pollutants into a unidirectional wind blowing in a domain of infinite extent (see illustration is figure X). The advection-diffusion equation can be derived by the mass conservation equation for the pollutant concentration C [kg/m3]:

∂C∂t+ ∇∙J=S

﻿where S [kg/m3s] is a source or sink term and the vector function J represents the mass flux [kg/m2s] of a pollutant owing to the combined effects of diffusion and advection. ﻿The diffusive flux (JD) is proportional to the concentration gradient through the diffusion/turbulence coefficient K [m2/s]:

JD=-K∇C

﻿The second contribution to the flux is due to simple linear advection by the wind, which can be expressed as:

JA=Cu

﻿Substitution X into the equation of conservation of mass (X) yields the three-dimensional advection-diffusion equation:

∂C∂t+ ∇∙Cu=∇∙K∇C+S

The Gaussian plume model, which is the solution of the three-dimensional advection-diffusion equation (x), eventually describes the pollutants’ concentration C [kg/m3] in a certain position in space:

Cx,y,z,Q=Quσyσz2πexp-y22σy2exp-z-H22σz2+exp-z+H22σz2

Where Q [kg/s] is the release rate from the stack, H [m] is the effective height (i.e., the sum of the actual stack height h and the plume rise h), x, y and z [m] are the downwind, crosswind and vertical distances, respectively, and u [m/s] is the mean wind speed at the height h of the release. σy and σz [m] are key parameters in the model and represent the standard deviation of the gaussian concentration distribution in the crosswind and vertical direction (figure X).

For simplicity, the steps for the solution of equation X were not specified. However, it is important to mention the simplifying assumptions that had to be made in order to reach the Gaussian plume model equation (X) [1]:

- The pollutant is emitted in a constant rate Q from a single point source located at height H above ground surface.

- The wind velocity is constant and aligned with the positive x-axis.

- ﻿The solution is steady state.

- ﻿The diffusion/turbulence coefficient K is a function of the downwind distance x only, and diffusion is isotropic so that Kxx=Kyx=Kzx=K(x).

- ﻿Wind velocity is sufficiently large that diffusion in the x-direction is much smaller than advection.

- ﻿Variations in topography are negligible so that the ground surface can be taken as the plane z=0.

- ﻿The pollutant does not penetrate the ground.

σ parameters are used instead of the diffusion/turbulence coefficient K(x), due to the fact that they are much easier to determine experimentally and can usually be described by a simple power law of the form: σy=axb, σz=cxd+f. This kind of dependence on downwind distance (x) expresses the widening of the plume when moving further from the source. ﻿Experimental measurements have been used to estimate the coefficients a, b, c, d and f under a variety of atmospheric conditions, and the most used values are those obtained by Turner 1994, using Pasquill-Gifford stability class categories [Pasquill, 1961]. These coefficients are specified in Table X.

In order to apply the suitable parameters to the Gaussian plume model, the Pasquill-Gifford stability category which characterizes the simulated set, should first be determined. The original method to do so, developed by Turner 1964, requires knowledge on cloud cover and cloud ceiling. Alternative methods were developed for situations where these data are not available. They include a radiation-based method which uses measurements of solar radiation during the day and delta-T at night and turbulence-based methods which use wind fluctuation statistics [Meteorological Monitoring Guidance]. For the current simulation, we chose to use a turbulence-based method (σA method), because of its simplicity and our currently available data. ﻿The σA method (sometimes referred to as σθ) uses the standard deviation of the wind direction in combination with the scalar mean wind speed, to determine the Pasquill-Gifford stability category (Tables X and X). Wind direction standard deviation values of 10-min resolution were obtained from the Israel Meteorological Service (See section data). To minimize the effects of wind meander (﻿long period oscillations associated with light wind speed conditions), it is recommended to calculate the 1-hour σA value using 10-min or 15-min averages, as specified in equation X (i.e., calculating the root mean square) [Meteorological Monitoring Guidance].

σθ1h=σθ1210min+σθ2210min+σθ3210min+σθ4210min+σθ5210min+σθ62(10min)6

Turbulence is essentially the motions of the wind over the time scales smaller than the averaging time used to determine the mean wind. Turbulence consists of circular whirls or eddies of all possible directions that serve to disperse pollutants by mixing with air having lower pollutant concentrations. It is common to divide the causes of turbulence to mechanical turbulence and buoyant generation of turbulence. Fluctuations in wind direction may represent mechanical turbulence that is formed by the friction with Earth’s surface. Distinguishing between daytime and nighttime takes into consideration buoyant turbulence generated by … all kinds of processes….NOT SURE ABOUT THIS

- Meteorology

Sources: Meteorology today, atmospheric chemistry and physics, workbook

As reviewed above, the meteorological parameters that play a role in the Gaussian plume model are wind speed, wind direction, and the atmospheric stability condition. The stability condition is a function of the vertical temperature profile of the atmosphere, which is govern by winds (cold or warm advection) or processes such as solar heating and radiative cooling. In general, three regimes of atmospheric stability exist: unstable, neutral and stable. In an absolutely unstable atmosphere, the environmental lapse rate (the rate at which air temperature changes with altitude) is greater than the dry adiabatic lapse rate (100 for every 1000 m), and an air parcel that starts to move upward will continue to rise because it is warmer and less dense than the air around it. Pollutants will be mixed rapidly in extreme vertical motions. In an absolutely stable atmosphere, the environmental lapse rate is less than the moist adiabatic rate (60 for every 1000 m) and an air parcel will resist an upward vertical motion and will tend to spread out horizontally. If the temperature of the atmosphere increases with altitude in a certain layer in the atmosphere, then this layer is called an inversion, which is a form of an absolutely stable atmosphere as well. Such conditions might lead to high pollution concentrations and minimal amount of dispersion. Between the above two extremes is the condition characterized as neutral. In this condition, the environmental lapse rate is characterized by a slight decrease of temperature with height, close to the dry adiabatic rate. These conditions can be caused for example by transitional situations near sunrise or sunset when the atmosphere changes its state from stable to unstable or vice versa. With neutral atmospheric conditions, intermediate level of dispersion takes place. It should be mentioned that meteorologists define a conditionally unstable atmosphere to distinguish between the absolutely stable and absolutely unstable conditions (instead of a neutral atmosphere). In this situation, the environmental lapse rate lies between the moist and dry adiabatic rates, and the stability of the atmosphere depends on whether the rising air is saturated or not. When the air parcel is not saturated, then the dry adiabatic lapse rate is the relevant reference state and the atmosphere is stable. For a saturated air parcel inside a cloud, the reference is the moist adiabatic rate and the atmosphere is unstable.

Add somewhere about day and night, also because of the PED separation.

Workbook +

- Evolutionary algorithms and Borg MOEA

אלגוריתם אבולוציוניים

כלי האופטימיזציה בורג

- Pairwise Euclidean distance (PED)

A criterion for evaluating how well a hypothetical network of sensors, placed in hypothetical locations, would be able to distinguish between overlapping plumes, was defined for the optimization problem. The criterion is the pairwise Euclidean distance (PED) between calculated readings of two sensor network sets with different number of active sources. That is, the PED of

two sets of active sources with different sizes {S}' and {S}" is:

PED{S'},{S''}=r=1|R|CrS'-CrS''2

In a generated network set that is be able to distinguish well between overlapping plumes (if given a source term estimation (STE) problem), a change in the number of active sources is expected to have a dramatic effect on the sensor network readings, resulting in high PED values. On the other hand, low PED values will characterize a network of sensors that will not be able to sufficiently resolve a problem of source separation and estimation. An example of a matrix of PED values is shown in Figure X for different combinations of 1-5 active sources. In our defined problem, we use the PED calculation as an objective function we wish to maximize. SEPARATE DAY AND NIGHT.

- Data

מטאורולוגי

For our simulation set, we used 10 min wind data obtained from the Israel Meteorological Service (IMS) for the years X and for one station Nevatim/Negev junction. This station was chosen since in future work, we may wish to implement our methodology on a real-world problem, possibly in an idealized location such as Ramat Hovav industrial zone in southern Israel.

Emission data was obtained from the Ministry of Environmental Protection….

These data were used for “evaluation” of our methodology…

- Methodology – problem formulation and optimization

התייחסות לסוג המזהם

- Simulation set (research plan?)

In our set, the simulated receptors (sensors) were taken at ground level, and so equation X was reduced to:

Cx,y,z,Q=Quσyσzπexp-y22σy2exp-H22σz2

גובה הארובה צריך להיות בגובה נתוני הרוח שאני לוקחת! למרות שנראה לי שיש הנחה שהרוח קבועה בכל גובה. 5 מטר זה בכל מקרה נמוך מידי ובגלל זה הריכוזים שלי לא הגיעו רחוק.

Our set-

- We take Z=0, so solution looks like:…

- Multiple sources (just the sum)

- Scale of simulation. no solution in 100 meters?

בעתיד(research plan?):

ולידציה לתוצאות

לקחת מקרה אמיתי (רמת חובב)

להשתמש במודל דיספרסיה מתוחכם יותר, אולי אין צורך?

להשתמש במקרה בעולם האמיתי. המעבר לעולם האמיתי הוא קשה. הוא דורש טופוגרפיה, כיסוי שטח, וידע על כל המקורות באזור המחקר וריכוזי רקע. וכנראה גם מודל מתוחכם יותר.

הדאטה של שיקגו

Initial results

A simulation using a synthetic set of stacks was performed in order to evaluate the proposed method for sensors’ placement. Usually, several constraints exist when placing static sensors; i) law requires sensors to be employed in populated areas, i.e., inside neighborhoods, ii) placing sensors inside the industrial region could have an effect on the network ability to separate between sources (?), iii) other physical constraints that may characterize the given study area (vegetation, public buildings).

This is especially important in the case of complex industrial site, where access to the leak source might require significant amount of resource such as trained personal, protection gear etc.

Basic idea:  using PED, using winds and atmospheric stability states that characterize the area.

Simulation 1: all sensors are the same and have perfect ( detection threshold is zero ?) sensitivity and infinite dynamic range (?).

Simulation 2: running a simulation for reasonable sets of detection thresholds and dynamic ranges. See what type of sensor suits.

Simulation 3: change the density of the stacks inside a defined area of constant size. Run for average yearly emissions Vs. table of factors emissions (my problem is with stacks that don’t work and then my PED calculation is for 5 and not less stacks).

Assumptions: the optimization process is based on average emission rates of the stacks.So basically we don’t take into consideration activity hours of the plants. These are indirectly considered during the optimization process by PED calculations. Technically we could correlate weather conditions with stacks’ working hours.

Such work may serve regulators and stakeholders.

What to display:

- Pollution dense map for an average wind direction, speed, and average emission rates of stacks for substance X.

- Distribution of wind speed and direction of the area.

- Distribution of stability classes.

- Assuming a sensor in each node of the grid, what is the distribution of my concentrations?

- Show a map of the weighted PED values (maybe higher resolution than the grid I run).

- Show a map of solutions for X sensors - an average of 10 runs. Is it clustered in groups?

- Perform random seed analysis

- Show the average pareto front of 10 runs.

- Show pareto fronts of different sensors types.

- Results for different density of stacks.

- What is the PED of my set of sensors I just chose, for different emission scenarios?

- להראות את מפת הריכוזים הממוצעים לעומת מפת הPED. לאו דווקא PED גבוה קורלטיבי עם ריכוזים גבוהים...

Work schedule

References

הערות

זה אולי לא המודל הכי מדויק אבל לצורך הבעיה שלנו הוא יכול להיות מספיק בהחלט בגלל שמסתכלים על ממוצעים שנתיים.

The pollutant concentrations in the plume are inversely proportional to the wind speed along the x axis, as can be seen from equation X. For example, if we double the wind speed, the concentration in the plume would be half of that with a lighter wind.

Incorporating considerations of both…

השיטה של מהירות הרוח בעצם לוקחת בחשבון יותר טורבולנציה מכאנית בפני הקרקע. החלוקה ליום ולילה

איורים

Pasquill stability class categories:

The criteria in Tables X and X are for data collected at 10 m and a roughness length of 0.15 m. Wind